Install OpenIPC in GOKE ( GK7205v210 ) - Windows 10

Minimum Hardware Required

1) UART to USB (FTDI)

2) Camera PCB (with Goke Processor)

3) Poe Cable & Power adaptor

Software Required (Install into your windows 10 )

1) PowerShell
2) Putty
3) Tftpd
4) winscp
5) Python and its dependencies ( progressbar , pyserial) 
Make sure environment variable  is updated for python. To verify the python installation, open PowerShell app and type “python –V” if it returns version then python is working fine in your system. You can install dependency by typing commands like “pip install progressbar” or “python -m pip install progressbar”

Steps to connect hardware

1) Find the Rx,Tx,G on PCB 

Here Red mark is Rx , Green is Tx and Blue is Ground
2) Solder one side of three wires (Jumper wires) on all these 3 points
If you are not perfect in soldering then try simple hack by using stapler pins (Shown in below pic). Pins should not touch each other.

3) Connect another sides of jumper wires into UART as shown in pic below.

Note: Switch Jumper into 3.3v (Marked in green arrow)
Connect Rx wire of PCB into Tx on UART and Tx of PCB into Rx on UART
4) Connect UART with Laptop by USB 
5) Connect LAN cable to camera (Assuming other side of LAN cable connected with router and your laptop is also connected with same router [with wifi or LAN]). Both laptop and camera should be connected in same local network


Steps at software side.

1) Open “System Device Manager” to check COM port number , to which UART connected.
 

Here it connected at COM10
2) Download openIPC burn files at https://github.com/OpenIPC/burn/archive/refs/heads/master.zip 
Unzip, rename it “burn” an place it location C:\
So it makes path like C:\burn\
Download u-boot file at https://github.com/OpenIPC/firmware/releases/
Download .bin file of your processor.
In my case it is https://github.com/OpenIPC/firmware/releases/download/latest/u-boot-gk7205v200-universal.bin
place u-boot-gk7205v200-universal.bin at location C:\burn\
You can also rename it like ubootgk7205v200universal.bin (Remove – sign)
Now u-boot file path is C:\ubootgk7205v200universal.bin

3) Download firmware
Visit https://openipc.org/supported-hardware/featured 
Click on docs icon of your processor
In my case it redirect to https://openipc.org/cameras/vendors/goke/socs/gk7205v210
Generate Macaddress, Select Memory chip, change camera and your laptop ip address.  Click on generate installation instructions.
Note: you can check laptop ip address by command ipconfig
To verify assigned ip to camera you can check it with their default firmware or by default app. In my case my camera support xmeye app.
I installed the app in my android phone (Phone is connected with same router by wifi) then find option to add camera then add camera by searching local network. It will show available device (camera) and its ip address.

Now scroll down the page (See below pic)

You will get download link of firmware. Download it and place the downloaded file at location c:\tftpd (Create tftpd folder if not exist)

4) Setup tftpd for file transfer.
Open tftpd application, go to setting, select base directory c:\tftpd 
Select TFTPD security “none”
Bind TFTPD to ip  address -> Select your laptop ip address

Click ok and it may ask to reload the application. Click Ok.
Now in main windows select correct ip address of laptop (Server interface option) 
Verify Current directory is c:\tftpd
Click on “Show Dir” button to verify that your firmware file is visible. Then click “Close” button.
See below pic.

Start writing firmware
1) Open two instances of power shell app
In first instance enter to the location c:\burn 
Type command 
cd C:\burn
Turn on the camera (Power on)
Then type command
python burn --chip gk7205v200 --file=ubootgk7205v200universal.bin -p COM10 --break
It starts copying u-boot files
Note: If it doesn’t start the copy (Showing trying to open COM10 then turn off-on camera power again – Without closing or cancel the command)
In case you got any error of json file then check json file data and correct it ..

If file has name of other json file then copy the data from that file and save it..

When it completed the copy then in 2nd instance of powerShell app run following command
Putty.exe -serial COM10 -sercfg 115200,8,n,1,N 
Terminal window of putty will popup to enter further commands.
You have to click on cursor and press enter key to get openipc#

Copy these commands which we generated on step “3) Download firmware”
# Enter commands line by line! Do not copy and paste multiple lines at once!

setenv ipaddr 192.168.1.104; setenv serverip 192.168.1.58
# in above commands ip should be correct, otherwise it will not copy the firmware files.

mw.b 0x42000000 0xff 0x800000

tftpboot 0x42000000 openipc-gk7205v210-lite-8mb.bin

sf probe 0; sf lock 0;

sf erase 0x0 0x800000; sf write 0x42000000 0x0 0x800000

reset

when you enter reset it reboot the camera and you have to immediately  interrupt the u-boot by pressing ctr + c key.
Then type command 

run setnor8m

If it ask you to login then it mean
OpenIPC installed!
Default username is root
Password is 12345

To open the web view type url in browser like: 
http://ipAddressOfCamera:85
Note: Ip address may be changed when openIPC installed so to verify the current ip address. Type ip address in terminal and check ipv4 ip